🧪 Lab: Exploring an ASA Firewall (GUI Only)

🎯 Objectives

- Understand how a firewall controls traffic

- Configure ASA

- Test connectivity before and after rules

🧰 Equipment

- Cisco Packet Tracer

- 1 ASA 5505

- 3 PCs

- (Optional) 1 switch

🌐 Topology

PC1,0 (Outside) → ASA → PC2 (inside)

Step 1: Configure Interfaces

Go to Interfaces

- Configure a logical VLAN 1 interface for the inside network (192.168.1.0/24) and set the security level to the highest setting of 100.

CCNAS-ASA(config)# interface vlan 1

CCNAS-ASA(config-if)# nameif inside

CCNAS-ASA(config-if)# ip address 192.168.1.1 255.255.255.0

CCNAS-ASA(config-if)# security-level 100

- Configure a logical VLAN 2 interface for the inside network (10.10.10.1.0/24) and set the security level to the highest setting of 100.

CCNAS-ASA(config-if)# interface vlan 2

CCNAS-ASA(config-if)# nameif outside

CCNAS-ASA(config-if)# ip address 10.0.0.1 255.255.255.0

CCNAS-ASA(config-if)# security-level 0

Step 2: Assign Ports

Go to Ports (Ethernet)

- Ethernet0/1 → VLAN 1 (inside)

- Ethernet0/0 → VLAN 2 (outside)

💻 Part 3: Configure PCs

PC2 (Inside)

- IP: 192.168.1.10                                   &                   Gateway: 192.168.1.1

PC1 (Outside)

- IP: 10.0.0.10                                         &                    Gateway: 10.0.0.1

🔍 Part 4: Test Connectivity (Before Rules)

- Click PC2 → Desktop → Command Prompt

- Ping PC1: ping 10.0.0.10

👉 Expected: ❌ It may fail (depending on Packet Tracer behavior)

🔓 Part 5: Allow Traffic

conf t
ciscoasa(config)#class-map inspection_default
ciscoasa(config-cmap)#match default-inspection-traffic
ciscoasa(config-cmap)#exit
ciscoasa(config)#policy-map global_policy
ciscoasa(config-pmap)#class inspection_default
ciscoasa(config-pmap-c)#inspect icmp

ciscoasa(config-pmap-c)#service-policy global_policy global

Test Again:

Ping from PC2 to PC1

👉 Expected: ✅ Works